|  | Stirling Engine HB26 |
| --- | --- |

Tools and Supplies

For assembling this kit, you need the following tools and supplies,

which are not included: Hex screwdriver Size 1.5, tweezers, pointed pliers, hex key Size 2.5,

Torx screwdrivers Size 8, 10 and 20.

Included are: 1 syringe of ceramic paste, 1 syringe of Aeroshell

Fluid 12 oil, 1 syringe of ball bearing adhesive Super Glue.

Assembly

1 – Carefully push one bearing (2) each into the sides of rocker arms (1).

2 – Using Loctite, glue one bearing (3) each into the side frames (9 + 10) – flush with the outside frame surface (countersunk). Apply just a small quantity of Super Glue to the bores of the side frames.

3 – Press the piston covers (20) into the displacer pistons (19), then screw the pistons (19) onto the displacer shafts (18). Apply a small amount of glue to the thread and carefully wipe off any excess adhesive with a cloth.

4 – Guide the displacer piston assemblies (19/20) into the cooling cylinders (22). If the cylinders are held in a vertical position, the displacer pistons must glide in by themselves and must not bind.

5 – Position the heating cylinders (23) over the cooling cylinders (22) and secure them with screws (24).

6 – On the inside of the rear side frame (10), guide the crankshaft (13) with its longer end into its bearing.

7 – Position one cooling cylinder (22) each on the inside of the side frames (9 + 10) and secure them with two Torx screws (29) each.

8 – Using pins (16), connect the short connecting rods (21) of crankshaft (13) to the displacer shafts (18) of cooling cylinders (22).

CAUTION: Do not cause damage to the surfaces of the shafts!

9 – Using a Torx screw (29), attach the rocker arm shaft (14) to the inside of the rear side frame (10).

10 – Slide on the rocker arms (1) with the aluminum part toward the front side frame (9).

11 – Using a Torx screw (8), attach the hex distance piece (4) to the rear side frame (10). The threaded holes of the hex distance piece (4) must be vertically aligned.

12 – Fit the front side frame (9) to the crankshaft (13), the rocker arm shaft (14), and the hex distance piece (4). Secure the assembly with Torx screws (8 + 29). When placed on an even surface, the assembled engine must not rock or tilt – the two side frames (9 + 10) must be in alignment.

13 – Attach the long connecting rods (27) of crankshaft (13) to rocker arms (1). Fit the black plastic bushings (11) into the connecting rods (27) and secure them from up front with screws (6). Tighten lightly with a torque of just 3 cNm to avoid squeezing the plastic bushings!

14 – Connect the short connecting rods (15) to working pistons (17) using pins (16).

CAUTION: The walls of the pistons are very thin, do not use pliers! One bore may be an easier fit than the other!

15 – Apply a touch of ceramic paste to your index finger and rub it against your thumb until the paste is virtually invisible. Now apply a thin coating to the working pistons (17). Guide the pistons (17) into the cooling cylinders (22) and move them back and forth. The pistons (17) must glide without scratching or binding! Make sure that there is no oil on working piston (17)! Using the syringe, apply a small drop of oil to the displacer shafts (18).

16 – Attach the connecting rods (15) and rocker arms (1). Fit the black plastic bushings (11) into the connecting rods (27) and secure them from up front with screws (6). Tighten lightly with a torque of just 3 cNm to avoid squeezing the plastic bushings!

17 – Turn the two grub screws (26) into the flywheels (25) – see assembly drawing.

18 – Fit a flywheel (25) to the front side frame (9) so that the crankshaft (13) is flush on the outside. Lightly tighten the grub screw (26).

19 – Fit the second flywheel (25) to the other side of the crankshaft (13) so that it butts against the bearing. Lightly tighten the grub screw (26).

20 – Fit the black drive wheel (30).

21 – Glue three rubber feet (31) to the underside of the beech base (12) – two to the outer corners of the rear edge and one centered along the front edge.

22 – Using screws (5) and washers (7), attach the engine to the beech base (12).

23 – Pull off the cover of burner (28), thread the wick through, then press the cover back on.

24 – Apply the self-adhesive Boehm logo.

25 – PLEASE NOTE: A slight unbalance of the flywheels (25) is normal and cannot be avoided. It is not covered by the warranty.

26 – Never loosen the grub screws of the crankshaft (13)! The crankshaft can only be aligned with special manufacturer’s tools!

Starting the Engine

CAUTION: The engine must be operated by or under the supervision of persons over 18 years of age.

Flammable objects must be kept from the vicinity of the engine.

Do not touch the engine since this could lead to serious burns.

Take great care when handling alcoholic spirits. Never leave spirits bottles open.

Inappropriate handling of the engine can cause fires! The legal regulations for open fires apply!

Instructions for Use

1 – Set up the engine in a draft-free area.

2 – Fill the aluminum burner cup to the lower marking with 94%

alcoholic spirits.

CAUTION: Alcoholic spirits may damage paints and lacquers. Close the spirits bottle tightly and store it in a secure place.

3 – Light the wick.

4 – Wait for approx. 10 seconds.

5 – Rotate the flywheels with force until the engine begins to run by itself.

• Never leave the running engine without supervision.

• With the burner positioned over to the far right, the engine will run more slowly.

• The black drive wheel can be used to drive accessories.

The HB26 has twice as much power as the HB25 engine and is ideal for powering accessories.

Care

The engine should be protected from dust. Even small particles can cause the engine to stop. All mechanical moving parts must be free and have play or the engine will not run!

Caution during dismantling: Many parts have a wall thickness of less 0.25 mm.

Maintenance:

After one Hour running time clean piston and Cylinder

Read SUPPORT instruction website

http://www.en.boehm-stirling.com/support.html

What if the engine will not start up?

• Check all mechanical components for free play.

• Have the black bushings been properly tightened?

• Has too much ceramic paste been applied?

• Has the working piston (17) become coated with oil?(after long downtime possible)

For technical advice, please contact:

E-mail: 	info@stirling-technik.de

Internet: 	www.stirling-technik.de

The Working Principles of a Stirling Engine

The burner heats the air, which is in a closed system. Due to the heat expansion of the air, the piston and the flywheels are put in motion. While the piston moves toward the flywheels, the displacer piston in the displacer cylinder is pushed into the cylinder head. Since the displacer piston does not have a seal, the hot air moves past it into the fin-cooled displacer cylinder. Here, the temperature is approx. 300 °C lower, the cooled air causes a vacuum, which pulls in the piston and keeps the flywheels turning. This rotary motion causes the displacer piston to be drawn back into the displacer cylinder, the cooled air rushes into the cylinder head. It heats up again, expands and thus provides power.